Optical alignment equipment for experimental tests

General information of the optical alignment equipment for experimental tests available:

- 3 vibration insulated optical tables.

- Different autocollimators:

  - NEWPORT LDS1000 (link) the LDS1000 electronic controller provides the means of displaying angular variations by measuring the movement of a spot of light on a position sensing device. General characteristics:

    - Digital resolution: 0.1 m.

    - Measurement range: ±2000 μrad.

    - Measurement error: ± (1 ±0.02 x measurement.) μrad (i.e. ±2%).

    - Max. measuring noise: 0.02 μrad/√Hz.

    - Sampling frequency: 2000 Hz.

    - Digital integration: from 0.5 msec to 1 sec.

  - ZG Optique AE-025 (link): the Electronic Autocollimator AE-025 is equipped with a digital camera of high resolution and allows the precise measurement of an angle position of moving and stationary objects.General information:

    - Aperture: 40 mm.

    - Autocollimator field of view hor.(x) x vert. (y): 2700 × 2160 arcsec.

    - Resolution: 0.01 arcsec.

    - Repeatability: 0.05 arcsec.

    - Accuracy: 0.25 arcsec.

    - Interface: USB 2.0.

    - Sample frequency = ~ 10Hz

  - Others (WILD T2, Taylor Hobson, Lunette Leitz)

- FISBA OPTIK AG interferometer (link): digital Compact Interferometer µPhase 2 HR S/N 814.

  - Laser 632.8 nm.

  - Resolution 1000 x 1000.

  - Internal reference flat at lambda/100.

  - Natural beam size: 5 mm.

  - 2 available objectives:

    - 100 mm.

    - 150 mm.

  - Non coated reference flat lambda/10 rms.

  - Non coated reference sphere r=-10 mm lambda/20.

  - 2 Coated reference flat 125 mm diameter at lambda/10 and faces parallel below 0.1 arcsec.

- Visible Spectrometer: Ocean Optics HR4000 (link). General characteristics:

  - Wavelength range: 200-1100 nm

  - Integration time: 4 milliseconds – 20 sec (continuous); 10 microseconds – 4 milliseconds (shutter)

  - Dynamic range: 3.4 x 106 (system); 1300:1 for single acqusition

  - Signal-to-noise ratio: 300:1 (at full signal)

  - Grating: H1 – H14; HC-1

  - Slit: 5 µm wide slits F#4

  - Optical resolution: ~0.2 nm FWHM

- Infrared Spectrograph: Yokogawa AQ6370C (link). General characteristics:

  - Wavelength range: 600 to 1700nm

  - High wavelength accuracy: ±0.01nm

  - High wavelength resolution: 0.02nm

  - Wide dynamic range: 78dB typ.

  - Wide level range: +20 to -90dBm.

  - Fast measurement: 0.2 sec. (100nm span).

- Visible and Infrared cameras, and detectors.

- Visible and IR laser sources.

- Different ultra bright white light sources:

  - Model EQ-99-FC LDLS system (link) consists of a Power Supply Controller unit, Lamp House, laser fiber optic cable, and Lamp House signal cable.

  - Supercontinuum White Light Source: LEUKOS – SM – OEM.

  - Others (HL-2000-FSHA lampe tungsten Ocean Optics, Laser Diode Thorlabs CPS192 780 nm, Diode laser Qphotonics QDFBLD-1550-5, etc)

- Integrating sphere: sphere of 280 mm of diameter in ODM98, from Gigahertz Optik (link). There are 2 FC fiber ports and an output port of 20 mm.